# METHOD OF STATEMENT

Date: 21st Nov 2023

Description of work

To shift out machines and accessories from production to loading area, using forklift and lorry crane to load up onto Malaysia lorry

Spec

2 unit Goodway GA200 / Wt: 3, 00 kg each

6 unit Chip Conveyor

1 unit CG 27-60

1 unit Hardford LG-500 CNC Milling / Wt: 3, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

27/11/23

Time of Operation

09:40

Location of Operation

#### 25 A.M.K St 65

Sanden International (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x 4.5ton forklift

Rollers/pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, riggers, signalman

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Hoist down forklift from lorry chassis

- To shift out machine from production to loading area

- Riggers to attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist up machine

- Load up machine onto Malaysia lorry

- Using of forklift to fork up and load accessories/machine onto Malaysia lorry chassis

- Secured machine with strapping belt

- Transport and deliver to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175