# METHOD OF STATEMENT

Date: 15th Aug 2022

Description of work

To shift out 1 unit machine & 1 unit Furnace to loading area, using forklift to fork up onto lorry chassis and deliver to another location

Machine Spec

Formech HD1500 Vacuum Foaming (to dismantle into 2 parts)

Dim 1: 290 x 160 x 240 cm / Wt: 2, 500 kg

Dim 2: 260 x 190 x 240 cm / Wt: 2, 500 kg

1 unit Furnace

Dim: 260 x 150 x 250 cm / Wt: 1, 800 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

24/08/22

Time of Operation

09:45 am

Location of Operation

#### Collection: 73 Nanyang Drive, Simtech Valley Block

#### Deliver: CT2B, Cleantech II, Block B Level 2

Equipment Use

1 x 633 lorry loader

1 x 4.5ton forklift

1 set wire/webbing sling

Pallet truck

Metal sheet/plywood, canvas (to lay out for floor protection)

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Hoist down forklift from lorry chassis

- To lay out metal plate along pathway for floor protection

- To shift out machine from production to loading area

- Using forklift to fork up and load machine onto lorry chassis

- Secured machine with strapping belt

- Transport and deliver machine to new location

- Forklift to fork down machine from lorry chassis

- Unload machine to the ground

- Using pallet truck to transport machine into factory premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175