# METHOD OF STATEMENT

Date: 11th Mar 2020

Description of work

To shift out 1 unit machine from production to loading bay, hoist up onto lorry chassis and deliver to scrapyard for dispose

Machine Spec

Mazak Integrex 300

Dim: 530 x 220 x 200 cm

Wt: 1, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

16/03/2020

Time of Operation

09:45 am

Location of Operation

#### 7 Loyang Way

NOV

Equipment Use

2 x 633 lorry loader

Hydraulic/hand jack

Woodblock

Rollers/pallet truck

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involve in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers under machine base

- Shift out machine from production to loading bay

- Insert sling to machine hoisting point

- Hoist up and load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of customer premise

- Deliver machine to scrapyard for dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175