# METHOD OF STATEMENT

Date: 02nd July 2019

Description of work

To shift out 4 unit machine from production room at ground floor to loading area, hoist up onto lorry chassis and deliver to scrapyard for disposal

Machine Spec

Fanuc Robodrill T14iB

Dim: 2000 x 1550 x 2400 mm

Wt: 2, 100 kgs each

Risk Assessment

A copy of Risk Assessment will be attach

Date Of Operation

04/07/2019

Time Of Operation

09:45 am

Location of Operation

#### No 31 Joo Koon Circle

Flextronics Manufacturing Singapore

Equipment Use

1-2 x lorry crane

Pallet truck

Crowbar/woodblock

1 set wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involve in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine to 3” high and insert woodblock under machine bed

- Insert pallet truck under machine bed for the move

- Shift out machine from production room to hoisting area

- Lorry crane to hoist up machine

- Load machine onto lorry chassis

- Secured machine with belt prior to moving out of customer premise

- Transport machine to scrapyard for disposal

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175