# METHOD OF STATEMENT

Reference No: EW/Q/0901/23

Date: 05th July 2023

Description of work

To shift out 1 unit machine from production to loading bay, hoist up onto lorry chassis and deliver to scrapyard for disposal

Machine Spec

1 unit DMC 835V CNC Milling

Dim: 240 x 220 x 265 cm

Wt: 6, 600 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

10/07/23 (Mon)

Time of Operation

09:45 am

Location of Operation

#### 2 Gul St 2

Supermec Pte Ltd

Equipment Use

1-2 x 633 lorry loader

Hand jack/crowbar

Woodblock

Rollers/pallet truck

1 set wire sling

1 x platform (to lay out for machine pathway)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involve in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers under machine bed

- Shift out machine from production into platform

- Insert sling onto crane hook

- Riggers to secured sling to rigging point of load to be lifted

- Lorry loader to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up and load machine onto lorry chassis

- Release all sling from rigging point of machine

- Secured machine with strapping belt prior to moving out of customer premise

- Transport machine to scrapyard for disposal

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175