# METHOD OF STATEMENT

Date: 12th Dec 2025

Description of work

To shift out 3 units machine from production to loading area, using forklift to fork up onto lorry chassis and deliver to Eric Wong warehouse

Machine Spec

3 units Injection machine

Wt: 7, 000 kg each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

31/12/25 – 02/01/26

Time of Operation

09:00 am

Location of Operation

3 Tuas Ave 2

Resmed Asia Pte Ltd

Equipment Use

2-3 x 633 lorry loader

1 x 10ton forklift

Rollers

Woodblock

Hydraulic/hand jack

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

Barricade operation area with safety cones, red/white tape and lifting in operation signage.

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers under machine bed

- Shift out machine from production area

- Transport machine to loading area

- Using 10ton forklift to fork up machine

- Load up machine onto lorry chassis

- Secured machine with strapping belt

- Transport and deliver machine to Eric Wong warehouse

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175