# METHOD OF STATEMENT

Date: 26th June 2023

Description of work

To shift out 1 unit machine from production room to loading area, using forklift to fork up and transfer to another factory

Machine Spec

1 unit Yasda YBM640V3

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

28/06/23 (Wed)

Time of Operation

10:00 am

Location of Operation

Fu Yu Corporation Limited

#### 8 Tuas Dr 1 – 9 Tuas Dr 1

Equipment Use

1 x 633 lorry loader

1 x 10ton forklift

Rollers/pallet truck

Woodblock

Hydraulic/hand jack

Canvas/metal plate (for epoxy floor protection)

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

Barricade operation area with safety cones, red/white tape and lifting in operation signage.

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers under machine bed

- Shift out machine from production area

- Transport machine to loading area

- Using 10ton forklift to fork up machine

- Transport machine to another factory

- Unload machine at door entrance

- Using rollers too transport into factory premise

- Position machine at the designated location

- Jack up, remove rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175