# METHOD OF STATEMENT

Date: 14th Aug 2023

Description of work

To shift out 1 unit machine from production room to loading area, using forklift to fork up onto low loader and deliver to scrapyard for disposal

Machine Spec

1 unit Toshiba BMC-80E

Dim: 650 x 330 x 320 cm

Wt: 18, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

TBA

Time of Operation

09:45 am

Location of Operation

10 Riverside Rd

Okamoto (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x 23ton forklift

1 x 12ft low loader

Rollers

Woodblock

Hydraulic/hand jack

Canvas/metal plate (for floor protection)

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

Barricade operation area with safety cones, red/white tape and lifting in operation signage.

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers under machine bed

- Shift out machine from production area

- Transport machine to loading area

- Using 23ton forklift to fork up machine

- Load up machine onto low loader

- Secured machine with strapping belt

- Transport and deliver machine to scrapyard for disposal

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175