# METHOD OF STATEMENT

Date: 16th Dec 2024

Description of work

To shift out machine from production to loading area, using forklift to fork up onto forwarder low bed

Spec

1 unit HAAS VR-4DAPC

Wt: 6, 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

18/12/24

Time of Operation

10:00 am

Location of Operation

#### 55 Loyang Dr

#### Asian Surface Technologies Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x 7ton forklift

Rollers/pallet truck

Woodblock

Hydraulic/hand jack

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Jack up machine using hydraulic jack

- Insert rollers under machine bed

- Shift out machine from production to loading area

- Using forklift to fork up machine

- Load up machine onto forwarder low bed

- Secured machine with strapping belt

- Transport and deliver machine to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175