# METHOD OF STATEMENT

Date: 22nd July 2024

Description of work

To shift out machine from workshop to loading area, using forklift to fork up onto lorry chassis and deliver to new location

Spec

1 unit 3D Printer machine

1 unit Sand Blast machine

3 cartons box of parts

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

07/08/24

Time of Operation

09:45 am

Location of Operation

#### NUS

#### 1 Engineering Drive 3, E3 Building

Equipment Use

2 x 633 lorry loader

1 x 3ton forklift

Wire/webbing sling

Pallet truck

Crowbar/woodblock

Canvas/PVC sheet (to lay out for floor protection)

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Rigger to insert sling to crane hook

- Secured sling to rigging point of forklift

- Hoist down forklift from lorry chassis to the ground

- To shift out machine from workshop to loading area

- Using forklift to fork up machine onto lorry chassis

- Secured machine with strapping belt

- Transport and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175