# METHOD OF STATEMENT

Date: 12th June 2023

Description of work

To shift out equipment from production to loading bay, load up onto lorry chassis and deliver to another location

Spec

4 unit Engine

Dim: 1.6 x 0.9 x 1.3m / Wt: 1, 000 kg each

2 unit Compressor

2 unit Air Tank

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

14/07/23 - to shift out 4 unit Engine

15/07/23 - to shift out 2 unit Compressor & 2 unit Air Tank

Time of Operation

09:45 am

Location of Operation

#### Collection

#### 2 Toh Tuck Link #03-01

Deliver

28 Ayer Rajah Crescent #04-

Equipment Use

1-2 x 633 lorry loader

1 x battery lifter

Pallet truck

Crowbar/woodblock

Obstacles

No obstacle on loading/unloading bay

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

# No operation will be carry out on heavy rain, thunderstorms and lighting weather

# All debris will be clear and disposed

# Safety Aspect

# All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in place

# All our workers are fully equip with standard P.P.E (safety shoe, safety helmet)

# Lifting machine and lifting gear with M.O.M certificate

# M.O.M certified forklift operator will be involved in the operation

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Shift out equipment from production to loading bay

- Hoist and load up equipment onto lorry chassis

- Secured with strapping belt prior to moving out of customer premise

- Transport and deliver to new location

- Hoist down and unload equipment from lorry chassis

- Insert pallet truck under equipment base

- Shift and transport to 4th floor via cargo lift

- Position equipment at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175