# METHOD OF STATEMENT

Date: 18th Aug 2023

Description of work

To shift out 1 unit printer from production to loading bay, using forklift to load up onto lorry and deliver to another location

Spec

UV Printer machine

Dim: 250 x 140 x 140 cm

Wt: 600 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

22/08/23

Time of Operation

09:45 am

Location of Operation

#### Collection

#### 438A Alexandra Rd #02-01

Deliver

28 Ayer Rajah Crescent #03-03

Equipment Use

1 x 633 lorry loader

1 x forklift

Pallet truck

Crowbar/woodblock

Obstacles

No obstacle on loading/unloading bay

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

# No operation will be carry out on heavy rain, thunderstorms and lighting weather

# All debris will be clear and disposed

# Safety Aspect

# All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in place

# All our workers are fully equip with standard P.P.E (safety shoe, safety helmet)

# M.O.M certified forklift operator will be involved in the operation

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Insert pallet truck under printer bed

- Shift out printer from 2nd floor to loading bay via cargo lift

- Using forklift to load up printer onto lorry chassis

- Secured printer with strapping belt prior to moving out of customer premise

- Transport and deliver to new location

- Forklift to unload printer from lorry chassis

- Insert pallet truck under printer bed

- Shift and transport to 4th floor via cargo lift

- Position printer at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175