# METHOD OF STATEMENT

Date: 16th Aug 2023

Description of work

To shift out machine and equipment from production to loading bay, fork up onto lorry chassis and deliver to another location

Machine Spec

1 unit IDI Micro-SLM

1 unit Essentium HSE180-S

5 unit cabinet

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

07-08/09/23

Time of Operation

09:45 am

Location of Operation

#### Collection

#### 5 Clean Tech Loop, Blk B,

CT2A

Deliver

5 Clean Tech Loop

CT2B

Equipment Use

1 x 633 lorry loader

1 x forklift

Pallet truck

Crowbar/woodblock

5 x wooden pallet

Obstacles

No obstacle on loading/unloading bay

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

# No operation will be carry out on heavy rain, thunderstorms and lighting weather

# All debris will be clear and disposed

# Safety Aspect

# All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in place

# All our workers are fully equip with standard P.P.E (safety shoe, safety helmet)

# Lifting machine and lifting gear with M.O.M certificate

# M.O.M certified forklift operator will be involved in the operation

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Shift out machine from 3rd floor to loading bay via cargo lift

- Using forklift to fork and load up machine onto lorry chassis

- Secured with strapping belt prior to moving out of customer premise

- Transport and deliver machine to new location within 100m away

- Using forklift to fork down and unload machine from lorry chassis

- Insert pallet truck under machine base

- Shift and transport to 3rd floor via cargo lift

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175