# METHOD OF STATEMENT

Date: 15 Oct 2015

Description of Work

To lay down 1 unit Tester inside customer premise, put onto wooden pallet and secured.

Shift out, load onto lorry chassis and deliver to new location

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

15/10/2015 - 10:00 am

Location of Operation

#### NUS - Architecture Drive (School of Design & Environment)

Equipment Use

1 x lorry crane

2 x forklift

Wire/webbing sling

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

# JOB SCOPE

- Tied up wire/webbing sling attached to forklift fork onto tester lifting point

- Lift up tester to about 40 cm high

- Tied up wire/webbing sling to bottom of Tester and attached to 2nd forklift fork

- 2nd forklift to lift up Tester slowly while 1st forklift to lower down fork

- Repeat the process till the Tester in horizontal position

- Load Tester onto wooden pallet and secured

- Shift our Tester, load onto lorry chassis and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175