# METHOD OF STATEMENT

Date: 02nd May 2019

Description of work

Transport lorry loader to unloading bay, uncrate case, hoist down machine from lorry chassis, shift and position to the designated location

Machine Spec

1 x Buffalo Lathe Micro Panther 446

Dim: 195 x 232 x 211cm

Wt: 2, 800 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

13/05/2019   -   09:45 am

Location of Operation

#### 2 Engineering Drive 1A

NUS

Equipment Use

1 x lorry loader- PALFINGER PK50002EH

Pallet truck, rollers

Webbing sling

Shackle

Obstacles

Ensure no obstacles around operation area and route to machine new location

Barricade work area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorm and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor on site has to ensure that risk control are in placed

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry loader at the designated location

# Uncrate case

# Hoist up and detach machine from base

# Swing crane and lower down machine to near ground level

# Insert pallet truck/rollers under machine base

# Shift and transport machine into workshop

# Position machine at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175