# METHOD OF STATEMENT

Date: 29th June 2018

Description of work

Transport lorry crane to unloading bay, uncrate case, hoist down equipment from lorry chassis, shift and position to 3rd floor via cargo lift and position to the designated location

Equipment Spec

1 x case of GALILEO equipment

Dim: 235 x 100 x 90 cm

Wt: 410 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

04/07/2018   -   11:00 am

Location of Operation

#### NUS – 2 Engineering Drive, EW1-04-02

#### Department of Civil and Environment Engineering

Equipment Use

1 x lorry crane

Pallet truck, rollers

Webbing sling

Shackle

Obstacles

Ensure no obstacles around operation area and route to machine position

Barricade work area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorm and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor on site has to ensure that risk control are in placed

All our workers are equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Transport lorry crane to unloading area

# Park lorry crane at the designated location

# Uncrate case

# Lorry crane to hoist down equipment to unloading bay

# Insert pallet truck/rollers to equipment base

# Transport equipment to 3rd floor via cargo lift

# Shift and position equipment at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and dispose

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel:62824175