# METHOD OF STATEMENT

Date: 21 April 2016

Description of Work

To unload 6 unit Milling and 1unit Lathe machine from lorry chassis, shift and position into customer premise at ground floor

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

26/04/2016 – 10:00 am

Location of Operation

#### NUS, 9 Engineering Drive 1

Advanced Manufacturing Lab EA-01-05

Equipment Use

2 x lorry crane

Pallet truck

Wire sling/shackle

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Uncrate machine case

- Hoist up and detach machine from base

- Swing crane to loading area

- Lower down machine slowly

- Insert pallet truck below machine base for the move

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175