# METHOD OF STATEMENT

Date: 05th Sep 2017

Description of Work

To unload 1 unit Furnace from lorry chassis, shift and position into customer premise at ground floor

Machine Spec

1 unit Furnace

Dim: 990 x 128 x 800 mm

Wt: 470 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

08/09/2017   –   11:00 am

Location of Operation

#### NUS, 2 Engineering Drive 3

Facaulty Of Engineering, Block E3-01-01

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for unloading of machine

- Hoist down furnace from lorry chassis

- Lower down furnace to near ground level

- Insert pallet truck to furnace base for the move

- Shift and position machine to the designated location at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175