# METHOD OF STATEMENT

Date: 12th Dec 2017

Description of work

To hoist up 1 unit machine to 4th floor by using platform and mobile crane

Machine Spec

1 unit Focaseiki TP-750 CNC

Dim: 2760 x 1400 x 2762 mm

Wt: 4000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

Estimate Delivery

27-30/12/2017   –   09:30 am

Location of Operation

#### No 10 Tampines Drive 4 #04-01/02, JTC Space @ Tampines North

AISTEC Engineering Pte Ltd

Equipment Use

2 x 50 ton mobile

2 x lorry crane

1 x platform

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade operation area with safety cones, red/white tape and lifting in operation signage                                                         All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

# Lifting operation are supervised by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated hoisting area for the lift

- Hoist up platform to 4th floor opening

- Tied back platform to a competent point and secured using 50 ton mobile crane

- Use belt to secured front left and right of platform to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 4th floor

- Lower down machine onto platform

- Insert pallet truck/rollers to machine base

- Shift out from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175