# METHOD OF STATEMENT

Date: 17th Jan 2017

Description of work

To hoist up 10 unit machines to 2nd floor by using platform and mobile/lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

23-24/01/2017 – 12:00 pm

(2 day job)

Location of Operation

#### No 10 Tampines Drive 4 #01-01

(Bloomfoss Pte Ltd)

Equipment Use

1 x 50 ton mobile

1 x 50 ton lorry crane

Lorry crane

1 x platform

Wire Sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement within the vicinity                                                            All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 2nd floor opening

- Tied back platform to a competent point and secured using 50 ton lorry crane

- Use belt to secured front left and right of platform to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 2nd floor

- Lower down machine onto platform

- Insert pallet truck/rollers to machine base

- Shift out from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175