# METHOD OF STATEMENT

Date: 27th Dec 2016

Description of work

1) To hoist down 2 unit storage rack from 3rd floor to ground floor and put aside

2) To hoist up 1 unit machine from inside production area, load onto lorry chassis and deliver to scrap yard for dispose

Machine Spec

High Speed Rubber Hydraulic Molding HM-300

Dim; 2200 x 3600 x 2300 mm

Wt: 10, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

28/12/2016 – 10:00 am

Location of Operation

#### No 10 Tuas South St 5

CDI Multiseals Pte Ltd

Equipment Use

1 x 50 ton lorry crane

Wire sling/shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc…)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to enter production area for the lift

- Hoist up machine and load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of premise and deliver to scrap yard

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175