# METHOD OF STATEMENT

Date: 02 April 2015

Description of work

To shift out 1 unit machine from production room to loading area, hoist up, load onto lorry chassis and deliver to No 45 Pandan Rd

Machine Spec

1 unit MPI machine

Dim: 430 x 105 x 271 cm

Wt: 1,200 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

08/04/2015 – 09:30 am

Location of Operation

#### No 11 Calshot Rd

Singapore Aero Engine Services Pte Ltd

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

Passage way and working area to be keep clear of any obstruction

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crews

MOM certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc…)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger, signalman and lorry loader operator

# JOB SCOPE

- Lift up machine to 3” high

- Insert rollers/pallet truck to machine base

- Shift out machine from production room to hoisting bay

- Hoist up and load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175