# METHOD OF STATEMENT

Date: 21 May 2015

Description of work

To shift out 1 unit machine from production room to loading area, hoist up, load onto low-loader and transfer to new location

Machine Spec

1 unit Sachman CNC Milling, Model: ARAKOS 521

Wt: 18,000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

22/05/2015 – 10:00 am

Location of Operation

#### No 15 Chin Bee Drive

ST Kinetic

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

Rollers/pallet truck

Low-loader

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crews

MOM certified lifting supervisor, rigger, signalman and lorry/mobile crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc…)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger, signalman and lorry loader operator

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers to machine base

- Shift out machine from production room to hoisting bay

- Hoist up and load machine onto low-loader

- Secured machine with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175