# METHOD OF STATEMENT

Date: 03rd June 2016

Description of work

To unload 1 unit Horizontal Horning machine with accessories from 1 x 40ft container, shift and position into customer premise at ground floor

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

To be advised

Location of Operation

#### No 15 Chin Bee Drive

ST Kinetics Singapore Ltd

Equipment Use

1 x lorry crane

Wire/webbing sling

Pallet truck/rollers

Obstacles

Clear all obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc... safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

# Job Scope

- Lorry crane to park at designated location (back to back with container)

- Level lorry chassis with container

- Pull out machine from container and load onto lorry chassis

- Transport machine to loading area

- Hoist and detach machine from base

- Lower down machine to ground level

- Insert pallet truck/rollers to machine base

- Shift and position machine into customer premise at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175