# METHOD OF STATEMENT

Date: 08 June 2015

Description of work

To shift out machineries from production room to loading area, hoist up, load onto lorry /       low-loader and transfer to new location

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

From 22-26/06/2015 – 10:00 am

Location of Operation

#### From No 16 Benoi Sector – No 8 Tuas West Ave

#### From No 88 International Rd – No 8 Tuas West Ave

(Hunting Energy Services Pte Ltd)

Equipment Use

50 ton mobile crane

50 ton lorry crane

Lorry crane

Low loader

Rollers/pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc…)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine

- Insert rollers to machine base

- Shift out machine from production room to hoisting bay

- Hoist up and load machine onto low-loader/lorry

- Secured machine with strapping belt prior to moving out of premise to new location

- At new premise, hoist down machines from low-loader/lorry

- Insert rollers to machine base

- Shift and position machine to the designated location inside factory

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175