# METHOD OF STATEMENT

Date: 16th July 2018

Description of Work

1) To unload 1 unit machine from lorry chassis, shift and position into production at ground floor using rollers/pallet truck

2) To shift out 1 unit Lathe machine from production to loading bay, hoist up onto lorry chassis and deliver to scrap yard for dispose

Machine Spec

1 unit Harrison V390-1250 Conventional Lathe

Dim: 270 x 138 x 165 cm

Wt: 1, 800 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

28/07/2018   –   10:00 am

Location of Operation

#### No 161 Gul Circle

Honeywell Aerospace (S) Pte Ltd

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Uncrate machine case

- Hoist up and detach machine from base

- Swing crane to unloading area

- Lower down machine to near ground level

- Insert rollers/pallet truck to machine base

- Transport machine into production

- Shift and position machine to the designated location

- Unload from rollers and let it sit on machine plate

- To shift out one unit old lathe to loading area, hoist up onto lorry chassis and deliver to scrap yard

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175