# Method Of Statement

Date: 18th Apr 2017

Description of work

To unload 2 unit machine from 1 x 40ft HQ container, uncrate and hoist up to 2nd floor by using platform and lorry crane

Machine Spec

Roders RXP 400 DS

Dim: 2515 x 1930 x 2070 mm

Wt: 3, 200 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

Est: 25/04/2017 - 10:00 am

Location of Operation

#### No 2 A.M.K Drive – Blk H

ITE College Central

Equipment Use

1 x 50 ton lorry crane

1 x 45ton lorry crane

1 x platform

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc.. safety shoe, helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# Job Scope

- Lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 2nd floor opening

- Stabilize platform and ensure it is full rested on the opening slab

- Tied back platform to competent point and secured using 45  ton lorry crane

- Use belt to secure front left and right of platform to prevent platform swing outward

- 50 ton lorry crane to hoist up machine to 2nd floor

- Lower down machine onto platform

- Insert pallet truck/rollers to machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175