# Method Of Statement

Date: 24th March 2016

Description of work

To unload 15 unit machine from 4 x 40ft container, uncrate, hoist up to 5th floor, shift and position into premise

Machine Spec

15 unit Campro CPV-550

Dim: 2240 x 2670 x 3000 mm

Weight: 3,000 kgs each

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

29/03/2016 - 10:00 am

Location of Operation

#### No 2 A.M.K Drive

ITE College Central

Equipment Use

2 x 50 ton mobile crane

1 x platform

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc.. safety shoe, helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# Job Scope

- Mobile crane to park at designated hoisting area for the lift

- Hoist up platform to 5th floor opening

- Stabilize platform and ensure it is full rested on the opening slab

- Tied back platform to competent point and secured using 50 ton mobile/lorry crane

- Use belt to secure front left and right of platform to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 5th floor

- Lower down machine onto platform

- Insert pallet truck/rollers to machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175