# Method Of Statement

Date: 22nd March 2016

Description of work

To unload 41 unit Lathe machine from 3 x 40ft container, uncrate, hoist up to 6th floor, shift and position into premise

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

24/03/2016 - 10:00 am

Location of Operation

#### No 2 A.M.K Drive

ITE College Central / Block H

Equipment Use

1 x 50 ton mobile crane

Lorry crane

Wire/webbing sling

1 x forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc.. safety shoe, helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# Job Scope

- Mobile crane to park at designated hoisting area for the lift

- Forklift to unload machine from container and load to the ground

- Uncrate case

- Hoist and detach machine from base

- 50 ton mobile crane to hoist up machine to 6th floor

- Lower down machine onto opening the opening slab

- Insert pallet truck/rollers to machine base

- Shift and position machine into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175