# METHOD OF STATEMENT

Date: 05th May 2017

Description of work

To shift out and hoist down 1 unit Kawashima Horizontal Form machine from 5th floor to 2nd floor by using platform and mobile crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

12/05/2017   -   10:00 am

Location of Operation

#### No 2 Chia Ping Rd

Haw Par Healthcare Ltd

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

1 x platform

1 x scissor lifter

Pallet truck/rollers

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated hoisting area for the lift

- Hoist up platform to 5th floor

- Stabilize platform effectively and ensure it is fully rested on the parapet wall

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Shift out machine from production  and load onto scissor lifter

- Lift up scissor lifter to level with parapet wall

- Shift machine into platform

- Secured machine onto platform with strapping belt

- 50 ton mobile crane to hoist down platform to 2nd floor

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175