# METHOD OF STATEMENT

Date: 08th June 2016

Description of work

To shift out and load 1 unit machine onto low loader by using customer overhead crane

Machine Spec

1 unit VRX I-800T

Dim: 5.780 x 3.500 x 3.580 m

Wt: 20,390 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

09/06/2016 - 10:00 am

Location of Operation

#### No 21 Joo Koon Circle

Yamazaki Mazak Singapore Pte Ltd

Equipment Use

1 x overhead crane

1 x spreader bar

Wire sling

1 x low loader

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc…)

Lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Hoist up machine using wire sling and overhead crane

- Reverse low loader to loading bay inside factory

- Load machine onto low loader

- Secured machine with belt prior to moving out of customer premise and transport to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175