# METHOD OF STATEMENT

Date: 18th Apr 2023

Description of Work

To unload 37 cases of 1 unit machine from 4 x 40ft OT container, uncrate, assist engineer to install machine parts, after install, shift and position into production at ground floor

Machine Spec

1 unit Danobat Grinding machine / Danuni 4100

Refer to packing list attached

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Est: 9-11/05/23

Time of Operation

09:30 am

Location of Operation

#### 21 Loyang Crescent

Safran Landing Systems Services Singapore Pte Ltd

Equipment Use

1 x 110ton lorry loader

633 lorry loader

1 x forklift

Rollers/pallet truck

Wire/webbing sling

Woodblock

Hydraulic/hand jack

1 x platform (to lay out in front of glass panel opening for machine pathway)

Metal plate (to lay out in front of glass panel opening for machine pathway)

Metal sheet/canvas (to lay out for epoxy floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated unloading area for the lift

- Insert sling to crane hook and secured onto rigging point of load

- Hoist down machine from container and unload to the ground

- Unscrew all bolts and nuts

- Hoist up and detach machine from base

- Swing crane and unload machine onto the laid out platform

- To assist engineer to install machine parts

- After installing, using rollers to shift and transport machine into inside factory

- Position machine at the designated location

- Unload machine form rollers and insert levelling pad under machine base

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd / Tel: 62824175