# METHOD OF STATEMENT

Date: 29th Apr 2019

Description of Work

To unload 1 unit machine from 1 x 20ft OT container, assist engineer to install machine parts, after install, shift and position into premise at ground floor

Machine Spec

1 unit CJK 5116G VTL

Packing Dimension: 3400 x 2400 x 2000mm

Wt: 12, 430 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

08/05/2019   -   09:30 am

Location of Operation

#### No 21 Loyang Lane

Windsor Airmotive Asia Pte Ltd

Equipment Use

1 x 633 lorry loader - PALFINGER PK100002F

Rollers/pallet truck

Wire/webbing sling

Metal plate/canvas (for epoxy floor protection)

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location for the lift

- Open up container canvas using ladder

- Secured sling to machine hoisting point

- Hoist up machine from container and load onto lorry chassis

- Transport lorry to near roller shutter door

- Hoist down machine from lorry chassis to the ground

- Assist engineer to install machine parts

- After installing, shift and transport machine into inside factory

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175