# METHOD OF STATEMENT

Date: 29 Oct 2015

Description of Work

To unload 1 unit machine from lorry chassis, insert pallet truck to machine base, shift and position into customer premise at 4th floor via cargo lift

Machine Spec

1 unit ACC 12.24 DX Surface Grinder

Dim: 2738 x 1929 x 1808 mm

Wt: 3,150 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

04/11/2015 – 10:00 am

Location of Operation

#### No 23 Loyang Way

GE Aviation Services Operation LLP

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lifting

- Secured wire/webbing sling to lifting point of machine

- Hoist and detach machine from base

- Swing crane and lower down machine slowly to ground level

- Insert rollers/pallet truck to machine base for the move

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175