# METHOD OF STATEMENT

Date: 21 Aug 2014

Description of work

To shift out machines and accessories from production area to hoisting bay, hoist up onto lorry and deliver to new location

Risk Assessment

A copy of Risk Assessment will be attach.

Date / Time of Operation

Est Date: 2/09/2014 - 09:30am

Location of Operation

#### From No 24 Woodlands Walk – No 2 Woodlands Sector 1

(ST Aerospace Engines Pte Ltd)

- Equipment Use

Lorry crane

Low loader

Pallet truck/rollers

Wire/webbing sling

Forklift

*   Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally.

- Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather.

All debris will clear and dispose

- Lifting Crews

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness etc)

Lifting machine and lifting gear with M.O.M cert.

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Barricade operation area

- Move and push out machine from production area to hoisting bay

- Hoist up and load machine onto low loader/lorry

- Secured machine with strapping belt prior to moving out from old premise to new location

- At new location, lorry crane to hoist machine to 1st and 2nd level of building

- Insert pallet truck/rollers to machine base for the move

- Shift and reposition machine into production area at 1st and 2nd level of building

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175