# METHOD OF STATEMENT

Date: 29th June 2018

Description of work

To shift out 1 unit machine from production room to loading area, hoist up onto lorry chassis and deliver to another factory

Machine Spec

GLS-135 AS Profile Grinder

Dim: 270 x 225 x 225 cm

Wt: 3, 950 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

03/07/2018   –   09:45 am

Location of Operation

#### From 9 Sixth Lok Yang Rd   -   3 Sixth Lok Yang Rd

Singapore Kobe Pte Ltd

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Metal plate/canvas (for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine to 3” high

- Insert pallet truck to machine base

- Shift out machine from production room to hoisting bay

- Hoist up and load machine onto lorry chassis

- Deliver machine to another factory

- Unload machine from lorry chassis

- Insert pallet truck

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175