# METHOD OF STATEMENT

Date: 03rd July 2017

Description of work

1) To shift out 1 unit machine from production room to loading area, hoist up and load onto forwarder lorry

2) To uncrate, shift and position 1 unit new machine to profile grinder room

Machine Spec

Waida PGX 2000S Profile Grinder

Dim: 120 x 170 x 210 cm

Wt: 4, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

06/07/2017 – 10:00 am (Shift out 1 unit Waida PGX 2000S Profile Grinder and load onto forwarder lorry)

11/07/2017 – 10:00 am (Shift in and position 1 unit Waida CNC Profile Grinder into tool room)

Location of Operation

#### 3 Sixth Lok Yang Rd

Singapore Kobe Pte Ltd

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc…)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 4” high

- Insert pallet truck to machine base

- Shift out machine from production room to hoisting bay

- Hoist up and load machine onto forwarder lorry

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175