# Method Of Statement

Date: 09 Jan 2015

Description of work

To unload machine from lorry chassis, uncrate case, load machine onto platform and hoist up to 2nd floor, shift and position into premise

Machine Spec

1 unit X087 CMM machine

Dim: 270 x 156 x 293 cm

Weight: 3,000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

To be advised

Location of Operation

#### No 31 Corporation Road

ST Electronic

- Equipment Use

1 x 50 ton lorry crane

1 x platform

Wire/webbing sling

Shackle

*   Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally.

- Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather.

All debris will be clear and dispose

- Lifting Crews

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc…safety shoe, helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# Job Scope

- Lorry crane to park at designated hoisting area for the lift

- Barricade operation area

- Hoist down crate from lorry chassis

- Uncrate case

- Load machine onto platform

- Fastened down and secured machine with strapping belt

- Hoist up platform together with machine to 2nd level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Insert rollers/pallet truck to below machine base

- Push and shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175