# METHOD OF STATEMENT

Date: 13th July 2017

Description of work

To shift out 1 unit machine and accessories from production room to loading area, hoist up and load onto wooden base for packing, after packing load onto container

Machine Spec

1 unit SL-403/2000

Dim: 580 x 260 x 260 cm

Wt: 14, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

Estimate: 19/07/2018   -   10:00 am

Location of Operation

#### No 34 Benoi Rd

#### Hunting Energy Services Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

1 x forklift

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers to machine base

- Shift out machine from production

- Transport machine to designated hoisting/packing area

- Hoist up machine and load onto wooden base for packing

- After packing, hoist up and load case onto FR container

- Secured case with lashing belt and choking wood

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175