# METHOD OF STATEMENT

Date: 12th Dec 2019

Description of work

To shift out 2 unit machine and accessories from production room to loading area, hoist up and load onto container

Machine Spec

1 unit Hwa Cheon Hi Tech 450AL

Dim: 430 x 200 x 200 cm

Wt: 6, 500 kgs

1 unit Kingston HP300 Lathe (From WYN2000 warehouse)

Dim: 570 x 200 x 190 cm

Wt: 9, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

14/12/2019

Time of Operation

09:45 am

Location of Operation

#### No 34 Benoi Rd

#### Hunting Energy Services Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x forklift

Rollers/pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers to machine base

- Shift out machine from production

- Transport machine to designated hoisting area

- Hoist up machine onto lorry chassis

- Reverse lorry crane to near container (back to back)

- Use outriggers jack to level up lorry with container

- Hoist up machine to near container door

- Lower down machine and insert rollers or metal rod under machine base

- Push and position machine into container

- Jack up machine and pull out rollers/metal rod

- Secured machine with lashing belt and choking wood

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175