# METHOD OF STATEMENT

Date: 09th Jan 2018

Description of work

To shift out 1 unit machine from production room to loading area, hoist up and load onto wooden base for packing, after packing load forwarder lorry

Machine Spec

1 unit HAAS TM3 CNC Milling

300 x 200 x 230 cm

Wt: 3, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

10/01/2018   -   10:00 am

Location of Operation

#### No 34 Benoi Rd

#### Hunting Energy Services Pte Ltd

Equipment Use

1 x 35/45 ton lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine to 3” high

- Insert rollers/pallet truck to machine base

- Shift out machine from production to designated hoisting/packing area

- Hoist up machine and load onto wooden base for packing

- After packing, load case onto container

- Secured case with choking wood and strapping belt

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175