# METHOD OF STATEMENT

Date: 25th Aug 2017

Description of work

To shift out 1 unit Mazak Integrex J-200 and 1 unit Everox H840 X3100 machine from 8 Tuas West Ave, hoist up, load onto lorry/low-load and transfer to new location at 34 Benoi Rd

Machine Spec

Mazak Integrex J-200

Dim: 300 x 240 x 250 cm

Wt: 7, 700 kgs

Everox H840 X3100 Lathe

Dim: 520 x 140 x 160 cm

Wt: 6, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

28/08/2017  –  10:00 am

Location of Operation

#### From No 8 Tuas West Ave – No 34 Benoi Rd

(Hunting Energy Services Pte Ltd)

Equipment Use

1 x 50 ton lorry crane

Lorry/low loader

Rollers/pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine

- Insert rollers to machine base

- Shift out machine from production room to hoisting bay

- Hoist up and load machine onto low-loader/lorry

- Secured machine with strapping belt prior to moving out of premise to new location

- At new premise, hoist down machines from low-loader/lorry

- Insert rollers to machine base

- Shift and position machine to the designated location inside factory

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175