# METHOD OF STATEMENT

Date: 06 Oct 2015

Description of work

To unload 4 case of 2 unit machine from 1 x 40ft container, shift and position into temporary location without uncrating

On another day, shift out the above cases, uncrate, shift and position into customer premise

Machine Spec

2 x NLX2500/700 CNC

Dim: 4230 x 2160 x 2150 mm

N/W: 5,200 kg each

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

09/10/2015 – unload from container and position into temporary location

To be advised – Uncrate, shift & positioning machine to location

Location of Operation

#### No 42B Penjuru Rd

Mencast Engineering Pte Ltd

Equipment Use

1 x lorry crane

Wire/webbing sling

Rollers/pallet truck

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc…safety shoe, helmet)

Lifting machine and lifting gear with M.O.M certificate

# Job Scope

- Lorry crane to park at designated location (back to back with container)

- Pull out cases from container onto lorry chassis

- Unload cases from lorry, shift into temporary storage area without uncrating

- On moving day, shift out cases from temporary area

- Uncrate cases

- Hoist and detach machine from base

- Insert rollers/pallet truck to shift and position machine into premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175