# METHOD OF STATEMENT

Date: 15th May 2017

Description of Work

To shift out 1 unit Makino RMC55, 1 unit Shzuoka Milling, 3 unit Granite Table, 4 unit small Granite and work table from tool room to loading area, hoist up and load onto lorry chassis, secured, transport to new location, unload, shift and position into customer premise at ground floor

Machine Spec

1 x Makino RMC 55

Dim: 210 x 180 x 210 cm   /   Wt: 2, 500 kgs

1 x Shzuoka NC Milling

Dim: 240 x 185 x 200 cm   /   Wt: 2, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

18/05/2017 – 10:00 am

Location of Operation

#### From No 5 Engineering Drive Blk EA Room E4-B1-04   To

Blk EA, Room EA-03-02 ground floor

Equipment Use

1 x lorry crane

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

# JOB SCOPE

- Lorry crane to park at designated hoisting area

- Shift out machine to loading area

- Hoist up machine and load onto lorry chassis

- Secured machine with strapping belt

- Transport machine to new location

- Unload machine from lorry chassis

- Insert pallet truck  for the move

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175