# METHOD OF STATEMENT

Date: 04th Oct 2019

Description of Work

To unload 2 packages of 1 unit machine from lorry chassis, uncrate, shift and position into customer premise at ground floor

Machine Spec

1 unit NXV600A

Package 1: 298 x 215 x 255 cm

Wt: 3, 300 kg

Package 2: 140 x 56 x 77 cm

Wt: 210 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

09/10/2019

Time of Operation

10:00 am

Location of Operation

#### 5 Tuas Dr 1

Fu Yu Corporation Limited

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location

- Insert sling to machine hoisting point

- Hoist up and detach machine from base

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine bed

- Transport machine into premise

- Shift and position machine at the designated location

- Unload machine from pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175