# METHOD OF STATEMENT

Date: 17th Sep 2018

Description of Work

To unload 4 cases of 1 unit machine from 1 x 40ft FR and 1 x 40ft HQ container, uncrate, shift and position into customer premise at ground floor

Machine Spec

1 unit H8 machine

Case 1: 730 x 405 x 395 cm   /   Gross Wt: 35, 200 kgs / Net Wt: 30, 000 kgs

Case 2: 430 x 230 x 200 cm   /   Gross Wt: 3, 800 kgs

Case 3: 400 x 230 x 210 cm   /   Gross Wt: 3, 000 kgs

Case 4: 300 x 225 x 225 cm   /   Gross Wt: 2, 350 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

Est: 24/09/2018   -   09:45 am

Location of Operation

#### 5 Tuas Dr 1

Fu Yu Corporation Limited

Equipment Use

1 x 100 ton mobile crane

1 x 50 ton lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated location for the lift

- Hoist down case from FR container to the ground

- Uncrate case

- Secured sling to machine hoisting point

- Hoist up and detach machine from base

- Swing machine to unloading area

- Lower down machine to near ground level

- Insert rollers to machine base

- Transport machine into premise

- Shift and position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175