# METHOD OF STATEMENT

Date: 23rd July 2018

Description of work

To shift out 1 unit machines and parts from production room to loading bay, hoist up and load onto 2 x M’sia lorry

Machine Spec

1 unit Toshiba BTD-11ER16

Main Bed Dim: 540 x 240 x 80 cm

Wt: 8, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date Of Operation

30/07/2018   -   Assist engineer to dismantle machine parts using customer 5 ton overhead crane

31/07/2018   -   Shift out machine and parts to loading bay, hoist up and load onto M’sia lorry

Time Of Operation

10:00 am

Location of Operation

#### No 5 Tuas Drive 1

Fu Yu Corporation Limited

Equipment Use

1 x 50 ton lorry crane

Heavy duty rollers

Pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Shift out machine and parts from production to loading bay

- Lorry crane to hoist up and load machine onto M’sia lorry

- Secured machine with strapping belt prior to moving out of customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175