# METHOD OF STATEMENT

Date: 09 September 2015

Description of Work

To unload 3 pallet of 1 unit 20 Lapping machine from lorry chassis, shift and position into customer premise

Risk Assessment

A copy of Risk Assessment will be attach.

Date / Time of Operation

10/09/2015 – Afternoon slot

Location of Operation

#### No 50 Nanyang Drive – Block S2 .2 Level B6-04

Nanyang Technology University

Equipment Use

1 x lorry crane

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crews

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Barricade operation area

- Hoist down machine from lorry chassis and load onto the ground

- Insert pallet truck below machine base for the move

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175