# METHOD OF STATEMENT

Date: 12 Nov 2015

Description of Work

To upright column base frame, load and assemble onto main machine bed underframe

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

To be advised

Location of Operation

#### No 51 Calshot Rd

Eagle Services

Equipment Use

1 x 75/50 ton lorry crane

1 x 4.5 ton forklift

Wire sling

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

# JOB SCOPE

- Shift column to near crane

- Tied wire sling to column hoisting point

- Hoist up column slowly

- Insert rollers to column base

- Secured wire sling to column base and attached to forklift

- Forklift to pull column while crane to lower down column slowly

- Repeat the process till the column in vertical position

- Hoist and load column onto machine bed to assemble

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175