# METHOD OF STATEMENT

Date: 30 July 2015

Description of work

To shift out machineries from production room to loading area, hoist up, load onto lorry chassis and transfer to new location

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

31/07/2015 – 10:00 am

Location of Operation

#### From No 6 Joo Koon Circle – No 4 Joo Koon Circle

ACP Metal Finishing Pte Ltd

Equipment Use

Lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc…)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Shift out machine from production room to loading bay

- Hoist up and load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of premise to new location

- At new premise, hoist down machines from lorry chassis and load onto ground

- Insert rollers/pallet truck to machine base

- Shift and position machine to the designated location inside factory

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175